## 1. Overview

This document summarises the work completed so far on our Feature Extraction part for a driver drowsiness detection system using CARLA.

We have:

- Implemented data capture within CARLA's 0.9.13 version, keeping the vehicle at 15km/hr constant and only controlling left and right.

- Extracted key features from steering angle and lateral offset.

- Built and evaluated a drowsiness classification pipeline(offline).

- Conducted offline analysis comparing *normal*, *moderate*, and *drowsy* driving states.

## 2. Data Collection

- Recorded 5 minutes of driving data per condition (awake, moderate fatigue, drowsy) across 1 participant.

- Logged CSV fields:

- `timestamp`

- `steering_angle_rad(Carla.VehicleControl.Steer)(which I don’t think so is correct)

- `lateral_offset`

- `lane_id`

## 3. Feature Extraction

Implemented Python scripts to compute:

1. **Steering Metrics**

- Mean, STD, Min, Max, Median

- Skewness, Kurtosis

- Shannon Entropy

- Reversal rate (per minute)

2. **Lateral and Lane Metrics**

- Standard deviation of lateral position (SDLP)

- Lane departure frequency (Hz)

- Lane keeping ratio

- Sliding‐window extraction (5 s windows) for real‐time feature streams, but done in an offline manner.

4. Offline Analysis

- `drowsiness_analysis.py`:

- Reads CSVs for *normal*, *moderate*, *drowsy* sessions.

- Computes all above features per session.

- Generates comparative boxplots for steering angle and lateral offset.

- Summary CSV exported with rows for each condition.

## 4. Next Steps

1. **Data Expansion**: Collect 10 min per condition for ≥ 10 or 5 drivers.

2. **Model Tuning**: Retrain with a larger dataset, experiment with additional features (i.e. Camera Module)

3. Validating our model is working properly and is able to recognise the driving pattern.